TABLE OF CONTENTS

ramdrive.device/CMD_CLEAR

ramdrive.device/CMD_READ

ramdrive.device/CMD_UPDATE

ramdrive.device/CMD_WRITE

ramdrive.device/KillRAD

ramdrive.device/KillRAD0

ramdrive.device/TD_CHANGENUM

ramdrive.device/TD_CHANGESTATE

ramdrive.device/TD_FORMAT

ramdrive.device/TD_GETGEOMETRY

ramdrive.device/TD_MOTOR

ramdrive.device/TD_PROTSTATUS

ramdrive.device/CMD_CLEAR                           ramdrive.device/CMD_CLEAR

NAME

CMD_CLEAR/ETD_CLEAR -- mark the internal buffer as containing

invalid data.

FUNCTION

As the ramdrive.device does not have an internal buffer, this

command performs no operation. It is only implemented to be

consistent with the trackdisk.device type of interface.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_CLEAR or ETD_CLEAR

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success.

SEE ALSO

CMD_WRITE, CMD_UPDATE

ramdrive.device/CMD_READ                             ramdrive.device/CMD_READ

NAME

CMD_READ/ETD_READ -- read data from ramdrive.

FUNCTION

These commands transfer data from the ramdrive to a supplied buffer.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_READ or ETD_READ

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to read

io_Offset	byte offset from the start of the card describing

where to read data from.

IO REQUEST RESULT

io_Error - 0 for success.

NOTES

SEE ALSO

CMD_WRITE

ramdrive.device/CMD_UPDATE                         ramdrive.device/CMD_UPDATE

NAME

CMD_UPDATE/ETD_UPDATE -- write out internal buffers to ram.

FUNCTION

As the ramdrive.device does not buffer data internally, this

command performs no operation and returns immediately. It is

implemented to support file systems that use it as part of the

regular trackdisk-type device interface.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_UPDATE or ETD_UPDATE

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success.

SEE ALSO

CMD_WRITE

ramdrive.device/CMD_WRITE                           ramdrive.device/CMD_WRITE

NAME

CMD_WRITE/ETD_WRITE -- write data to ramdrive.

FUNCTION

These commands transfer data from a supplied buffer to ram.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_WRITE or ETD_WRITE

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to write.

io_Offset	byte offset from the start of the card describing

where to write data to.

IO REQUEST RESULT

io_Error - 0 for success.

NOTES

SEE ALSO

CMD_READ, TD_FORMAT

ramdrive.device/KillRAD                               ramdrive.device/KillRAD

NAME

KillRAD -- kill ramdrive.device unit

SYNOPSIS

devName = KillRAD(unit)

char *KillRAD( ULONG );

FUNCTION

Kill a RAD: disk and return its memory.  First, remove the

system hooks that recover the disk memory after reset, then

free the disk memory, and return the dos device name.

INPUTS

unit - the unit of the ramdrive.device to kill.

RESULTS

devName - a pointer to the colon terminated device name that

was associated with this unit.  This is a null

terminated C string.

ramdrive.device/KillRAD0                             ramdrive.device/KillRAD0

NAME

KillRAD0 -- kill ramdrive.device unit 0

SYNOPSIS

devName = KillRAD(unit)

char *KillRAD( ULONG );

FUNCTION

Perform a KillRAD(0).  This function is retained for

historical reasons.

SEE ALSO

ramdrive.device/KillRAD

ramdrive.device/TD_CHANGENUM                     ramdrive.device/TD_CHANGENUM

NAME

TD_CHANGENUM -- return the current value of the counter.

FUNCTION

This command returns the current value of the ramdrive counter (as

used by the enhanced commands). The change counter is always 1.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_CHANGENUM

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success.

io_Actual - is always 1.

ramdrive.device/TD_CHANGESTATE                 ramdrive.device/TD_CHANGESTATE

NAME

TD_CHANGESTATE -- check if the ramdrive is currently available.

FUNCTION

This command checks to see if the ramdrive is available. As it

always is, this command always returns 0.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_CHANGESTATE

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success.

io_Actual - 0, indicating that the ram can be read from or

written to.

ramdrive.device/TD_FORMAT                           ramdrive.device/TD_FORMAT

NAME

TD_FORMAT/ETD_FORMAT -- Write data to memory

FUNCTION

This function is traditionally used to write data to a disk

that has not yet been formatted before or has had a hard error

on a standard write command. None of these states apply to the

ramdrive.device, and as such, this command is identical to

CMD_WRITE.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_FORMAT or ETD_FORMAT

io_Flags	0 or IOF_QUICK

io_Data		points to a buffer containing the data to write to the

track, must be at least as large as io_Length.

io_Length	number of bytes to write.

io_Offset	byte offset from the start of the device.

IO REQUEST RESULT

io_Error - 0 for success.

NOTES

SEE ALSO

CMD_WRITE, TD_RAWWRITE

ramdrive.device/TD_GETGEOMETRY                 ramdrive.device/TD_GETGEOMETRY

NAME

TD_GETGEOMETRY -- return the geometry of the ramdrive.

FUNCTION

This command returns a full set of information about the

layout of the drive. The information is returned in the

DriveGeometry structure pointed to by io_Data.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_GETGEOMETRY

io_Flags	0 or IOF_QUICK

io_Data		Pointer to a DriveGeometry structure

io_Length	sizeof(struct DriveGeometry)

IO REQUEST RESULT

io_Error - 0 for success.

NOTE

SEE ALSO

TD_GETDRIVETYPE, TD_GETNUMTRACKS

ramdrive.device/TD_MOTOR                             ramdrive.device/TD_MOTOR

NAME

TD_MOTOR/ETD_MOTOR -- control the on/off state of the virtual motor.

FUNCTION

This command would usually give control over the drive LED or

motor of a drive. As the ramdrive does not have either, this command

does nothing and always indicates success.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_MOTOR or ETD_MOTOR

io_Flags	0 or IOF_QUICK

io_Length	ignored.

IO REQUEST RESULT

io_Error - 0 for success

io_Actual - always 1, indicating a running motor.

ramdrive.device/TD_PROTSTATUS                   ramdrive.device/TD_PROTSTATUS

NAME

TD_PROTSTATUS -- return whether the ram is write-protected.

FUNCTION

This command is used to determine whether the ram drive is

currently write-protected. The write protection status can

be set with bit 0 of the flags value of OpenDevice().

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_PROTSTATUS

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success.

io_Actual - if io_Error is 0, this tells you whether the device

is write-protected. 0 means the device is NOT write-

protected, while any other value indicates it is.