TABLE OF CONTENTS

scsi.device/CMD_READ

scsi.device/CMD_START

scsi.device/CMD_STOP

scsi.device/CMD_UPDATE

scsi.device/CMD_WRITE

scsi.device/HD_SCSICMD

scsi.device/TD_ADDCHANGEINT

scsi.device/TD_CHANGENUM

scsi.device/TD_CHANGESTATE

scsi.device/TD_EJECT

scsi.device/TD_FORMAT

scsi.device/TD_FORMAT64

scsi.device/TD_GETGEOMETRY

scsi.device/TD_MOTOR

scsi.device/TD_PROTSTATUS

scsi.device/TD_READ64

scsi.device/TD_REMCHANGEINT

scsi.device/TD_SEEK

scsi.device/TD_SEEK64

scsi.device/TD_WRITE64

scsi.device/CMD_READ                                     scsi.device/CMD_READ

NAME

CMD_READ -- read sectors of data from a disk.

FUNCTION

This command reads data from the drive to a supplied buffer.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_READ

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to read, must be a multiple of

the drive block size.

io_Offset	byte offset from the start of the disk describing

where to read data from, must be a multiple of

the drive block size.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

This command can only reach data below the 4GB barrier. To read

data beyond the barrier, use TD_READ64 instead.

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to read more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

CMD_WRITE, TD_READ64

scsi.device/CMD_START                                   scsi.device/CMD_START

NAME

CMD_START -- start a drive, if possible.

FUNCTION

This command causes the drive to attempt spinning up the motor and

return from an idle operation.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_START

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

Drives may or may not react on this command. A possible

reaction is that a parked drive spins up its motor and is

then ready to take read or write commands.

BUGS

scsi.device/CMD_STOP                                     scsi.device/CMD_STOP

NAME

CMD_STOP -- stop a drive, if possible.

FUNCTION

This command causes the drive to attempt spinning down the motor,

park the drive heads and enter an idle state.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_STOP

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

Drives may or may not react on this command. A possible

reaction is that the drive spins down its motor and parks

its drive head. A CMD_START is then required to return the

drive into an operational stage.

BUGS

scsi.device/CMD_UPDATE                                 scsi.device/CMD_UPDATE

NAME

CMD_UPDATE -- write out drive cache to the disk.

FUNCTION

This command may instruct the connected drive to flush its internal

buffers to the disk, and hence complete a write operation. Whether

the drive actually has a drive cache, and performs any operation in

response of this command is implementation defined.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_UPDATE or ETD_UPDATE

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

SEE ALSO

TD_WRITE64, CMD_WRITE

scsi.device/CMD_WRITE                                   scsi.device/CMD_WRITE

NAME

CMD_WRITE -- write sectors of data to a disk.

FUNCTION

This command transfer data from a supplied buffer to a disk.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_WRITE

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to write, must be a multiple of

the disk block size.

io_Offset	byte offset from the start of the disk describing

where to write data to, must be a multiple of

the disk block size.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

This command can only reach data below the 4GB barrier. To write

data beyond the barrier, use TD_WRITE64 instead.

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to write more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

TD_WRITE64, CMD_READ, TD_FORMAT

scsi.device/HD_SCSICMD                                 scsi.device/HD_SCSICMD

NAME

HD_SCSICMD -- issue a SCSI command to the drive

FUNCTION

This command issues a SCSI command to the drive, bypassing the

translation from trackdisk to SCSI commands.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	HD_SCSICMD

io_Data		pointer to a struct SCSICmd

io_Length	sizeof(struct SCSICmd)

io_Flags	0 or IOF_QUICK

SCSI COMMAND INPUT

scsi_Data	word aligned buffer to be filled or

transmitted during the data phase of

the scsi command.

scsi_Length	number of bytes to be transfered during

the SCSI data phase

scsi_Actual	filled upon return with the actual number

of bytes transmitted

scsi_Command	pointer to a word aligned buffer containing

the bytes to be transmitted during the

SCSI command phase. That is, this buffer

contains the SCSI commands to be issued.

scsi_CmdLength	number of bytes to be transmitted during

the SCSI command phase, i.e. the byte size

of the command.

scsi_CmdActual	filled upon return with the actual number

of bytes transfered during the command

phase.

scsi_Flags	various flags, such as:

SCSIF_WRITE: transfer from RAM to disk

SCSIF_READ: transfer from disk to RAM

SCSIF_AUTOSENSE: automatically sense

error status upon failure

scsi_Status	returned status of the command, see

<devices/scsidisk.h>

scsi_SenseData	pointer to a word aligned buffer that

is filled with sense data upon failure

if the SCSIF_AUTOSENSE flag is set.

scsi_SenseLength size of the autosense buffer

scsi_SenseActual actual number of sense bytes returned

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h> or <devices/scsidisk.h>

NOTES

The scsi.device may actually not be connected to a SCSI

bus at all, but may emulate (some) SCSI commands through

appropriate IDE commands. If the connected device speaks

ATAPI, then the SCSI commands are directly dispatched

as ATAPI commands. Note that ATAPI uses the same command

set as SCSI.

However, for non-ATAPI devices, the following table lists

which SCSI commands are emulated through ATA commands:

READ(6)		(0x08)

READ(10)	(0x28)

WRITE(6)	(0x0a)

WRITE(10)	(0x2a)

REQUEST_SENSE	(0x03)

FORMAT_UNIT(6)	(0x04)

TEST_UNIT_READY	(0x00)

SEEK(6)		(0x0b)

SEEK(10)	(0x2b)

READ_CAPACITY	(0x25)

MODE_SENSE	(0x1a)

INQUIRY		(0x12)

REZERO_UNIT	(0x01)

VERIFY		(0x2f)

RESERVE		(0x16)

SEND_DIAGNOSTIC	(0x1d)

The following mode pages are emulated through MODE_SENSE:

Page 3 (Format Device page)

Page 4 (Rigid Disk page)

BUGS

scsi.device/TD_ADDCHANGEINT                       scsi.device/TD_ADDCHANGEINT

NAME

TD_ADDCHANGEINT -- add a disk change software interrupt handler.

FUNCTION

This command lets you add a software interrupt handler to the

scsi.device that gets invoked whenever a disk insertion or removal

occurs.

You must pass in a properly initialized Exec Interrupt structure

and be prepared to deal with disk insertions/removals

immediately. From within the interrupt handler, you may only call the

status commands that can use IOF_QUICK.

To set up the handler, an Interrupt structure must be initialized.

This structure is supplied as the io_Data to the TD_ADDCHANGEINT

command. The handler then gets linked into the handler chain and

gets invoked whenever a disk change happens. You must eventually

remove the handler before you exit.

This command only returns when the handler is removed. That is,

the device holds onto the IO request until the TD_REMCHANGEINT command

is executed with that same IO request. Hence, you must use SendIO()

with this command.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_ADDCHANGEINT

io_Flags	0

io_Length	sizeof(struct Interrupt)

io_Data		pointer to Interrupt structure

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

SEE ALSO

TD_REMCHANGEINT, <devices/trackdisk.h>, <exec/interrupts.h>,

exec.library/Cause()

scsi.device/TD_CHANGENUM                             scsi.device/TD_CHANGENUM

NAME

TD_CHANGENUM -- return the current value of the disk-change counter.

FUNCTION

This command returns the current value of the disk-change counter (as

used by the enhanced commands). The disk change counter is incremented

each time a disk is inserted or removed from the device unit.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_CHANGENUM

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

io_Actual - if io_Error is 0, this contains the current value of the

disk-change counter.

scsi.device/TD_CHANGESTATE                         scsi.device/TD_CHANGESTATE

NAME

TD_CHANGESTATE -- check if a disk is currently in a device unit.

FUNCTION

This command checks to see if there is currently a disk in a unit.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_CHANGESTATE

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

io_Actual - if io_Error is 0, this tells you whether a disk is in

the drive. 0 means there is a disk, while anything else

indicates there is no disk.

scsi.device/TD_EJECT                                     scsi.device/TD_EJECT

NAME

TD_EJECT -- eject (or load) the disk in the drive, if possible.

FUNCTION

This command causes the drive to attempt to eject the disk in

it, if any.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_EJECT

io_Flags	0 or IOF_QUICK

io_Length	0 (load, if supported) or 1 (eject)

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

BUGS

scsi.device/TD_FORMAT                                   scsi.device/TD_FORMAT

NAME

TD_FORMAT -- write data to disk.

FUNCTION

This command is traditionally used to write data to disk that

has not yet been formatted or has had a hard error on a standard write

command. However, as scsi and ide drives come pre-formatted, these

commands are identical to CMD_READ.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_FORMAT

io_Flags	0 or IOF_QUICK

io_Data		points to a buffer containing the data to write to the

disk, must be at least as large as io_Length.

io_Length	number of bytes to format, must be a multiple of the

block size of the drive.

io_Offset	byte offset from the start of the disk for the block

to write to, must be a multiple of the drive block size.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

This command can only reach data below the 4GB barrier. To write

data beyond the barrier, use TD_FORMAT64 instead.

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to write more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

CMD_WRITE, TD_FORMAT64

scsi.device/TD_FORMAT64                               scsi.device/TD_FORMAT64

NAME

TD_FORMAT64 -- write data to disk, 64 bit extended

FUNCTION

This command is traditionally used to write data to disk that

has not yet been formatted or has had a hard error on a standard write

command. However, as scsi and ide drives come pre-formatted, these

commands are identical to TD_WRITE64.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_FORMAT64

io_Flags	0 or IOF_QUICK

io_Data		points to a buffer containing the data to write to the

disk, must be at least as large as io_Length.

io_Length	number of bytes to format, must be a multiple of the

block size of the drive.

io_Offset	low 32 bits of the byte offset from the start of the

disk to the block to write to.

io_Actual	high 32 bits of the byte offset.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to write more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

TD_SEEK64

scsi.device/TD_GETGEOMETRY                         scsi.device/TD_GETGEOMETRY

NAME

TD_GETGEOMETRY -- return the geometry of the drive.

FUNCTION

This command returns a full set of information about the

layout of the drive. The information is returned in the

DriveGeometry structure pointed to by io_Data.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_GETGEOMETRY

io_Flags	0 or IOF_QUICK

io_Data		Pointer to a DriveGeometry structure

io_Length	sizeof(struct DriveGeometry)

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTE

This information may change when a disk in inserted when

certain hardware is present.

The information provided with this command may not actually

reflect the true device geometry as hard disk geometries are

more complex than what can be described by the DriveGeometry

structure. However, the size of the medium described by this

structure is "approximately correct" in the sense that the

sector size is correct (or accepted by the drive), and the

number of blocks due to this structure is not larger than

the physical block count of the drive.

SEE ALSO

TD_GETDRIVETYPE, TD_GETNUMTRACKS

scsi.device/TD_MOTOR                                     scsi.device/TD_MOTOR

NAME

TD_MOTOR/ETD_MOTOR -- control the on/off state of a drive motor

or the drive LED.

FUNCTION

This command gives control over the disk motor or the drive status

LED. The motor/motor light may be turned on or off.

Turning the drive on is not necessary, the device does this

automatically if it receives a request when the motor is off. Some

drives control the drive LED automatically ignoring the TD_MOTOR

command.

In addition, the standard instructions to the user are that it is safe

to remove a disk from a drive if and only if the motor is off (that is,

if the disk light is off).

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_MOTOR or ETD_MOTOR

io_Flags	0 or IOF_QUICK

io_Length	the requested state of the motor, 0 to turn the motor

off, and 1 to turn the motor on.

iotd_Count	(ETD_MOTOR only) maximum allowable change counter

value.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

io_Actual - if io_Error is 0 this contains the previous state of the

drive motor.

scsi.device/TD_PROTSTATUS                           scsi.device/TD_PROTSTATUS

NAME

TD_PROTSTATUS -- return whether the current disk is write-protected.

FUNCTION

This command is used to determine whether the current disk is

write-protected.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_PROTSTATUS

io_Flags	0 or IOF_QUICK

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

io_Actual - if io_Error is 0, this tells you whether the disk in the

drive is write-protected. 0 means the disk is NOT write-

protected, while any other value indicates it is.

scsi.device/TD_READ64                                   scsi.device/TD_READ64

NAME

TD_READ64 -- read sectors of data from a disk.

FUNCTION

This command reads data from the drive to a supplied buffer.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	CMD_READ

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to read, must be a multiple of

the drive block size.

io_Offset	low 32 bit of the byte offset from the start of the

disk describing where to read data from, must be a

multiple of the drive block size.

io_Actual	high 32 bit of the byte offset where to read data

from.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to read more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

TD_WRITE64, TD_WRITE64

scsi.device/TD_REMCHANGEINT                       scsi.device/TD_REMCHANGEINT

NAME

TD_REMCHANGEINT -- remove a disk change software interrupt handler.

FUNCTION

This command removes a disk change software interrupt added

by a previous use of TD_ADDCHANGEINT.

IO REQUEST INPUT

The same IO request used for TD_ADDCHANGEINT.

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_REMCHANGEINT

io_Flags	0

io_Length	sizeof(struct Interrupt)

io_Data		pointer to Interrupt structure

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

BUGS

SEE ALSO

TD_ADDCHANGEINT, <devices/trackdisk.h>

scsi.device/TD_SEEK                                       scsi.device/TD_SEEK

NAME

TD_SEEK -- control positioning of the drive heads with 64 bit offset

FUNCTION

This command is currently provided for internal diagnostics, disk

repair, and head cleaning only. Drives may or may not implement

them.

TD_SEEK move the drive heads to the track specified. The io_Offset

field should be set to the (byte) offset to which the seek is

to occur. TD_SEEK may not verify their position until the next

read. That is, they may only move the heads; they do not actually read

any data. Actually, some drives may do nothing.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_SEEK

io_Flags	0 or IOF_QUICK

io_Offset	byte offset from the start of the disk describing

where to move the head to. This shall be a multiple

of the drive block size.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

scsi.device/TD_SEEK64                                   scsi.device/TD_SEEK64

NAME

TD_SEEK64 -- control positioning of the drive heads.

FUNCTION

These commands are currently provided for internal diagnostics,

disk repair, and head cleaning only. Drives may or may not implement

them.

TD_SEEK64 move the drive heads to the track specified. The io_Offset

field should be set to the (byte) offset to which the seek is

to occur. TD_SEEK64 may not verify their position until the next

read. That is, they may only move the heads; they do not actually read

any data. Actually, some drives may do nothing.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_SEEK

io_Flags	0 or IOF_QUICK

io_Offset	low 32 bits of the byte offset from the start

of the disk describing where to move the head to.

This shall be a multiple of the drive block size.

io_Actual	high 32 bits of the byte offset to seek to.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

scsi.device/TD_WRITE64                                 scsi.device/TD_WRITE64

NAME

CMD_WRITE -- write sectors of data to a disk, with 64 bit offset

FUNCTION

This command transfer data from a supplied buffer to a disk.

IO REQUEST INPUT

io_Device	preset by the call to OpenDevice()

io_Unit		preset by the call to OpenDevice()

io_Command	TD_WRITE64

io_Flags	0 or IOF_QUICK

io_Data		pointer to the buffer where the data should be put

io_Length	number of bytes to write, must be a multiple of

the disk block size.

io_Offset	low 32 bits of the byte offset from the start of

the disk describing where to write data to, must be

the disk block size.

io_Actual	high 32 bits of the byte offset.

IO REQUEST RESULT

io_Error - 0 for success, or an error code as defined in

<devices/trackdisk.h>

NOTES

BUGS

Some incarnations of the scsi.device, namely those that handle

actually IDE devices, were unable to write more than 255 blocks

in one go. This bug has been fixed in V47.

SEE ALSO

TD_WRITE64, TD_READ64, TD_FORMAT64