TABLE OF CONTENTS

mmu.resource/--Background--

mmu.resource/AdjustCtxtSettings

mmu.resource/AdjustMMUContext

mmu.resource/BuildMMUConfig

mmu.resource/CheckMMUInterface

mmu.resource/CmpDescriptor

mmu.resource/DefineMMUContext

mmu.resource/Exception

mmu.resource/FindDescriptor

mmu.resource/ForbidDMACache

mmu.resource/IncrementDescriptor

mmu.resource/InstallNewPage

mmu.resource/InstallPage

mmu.resource/LoadRoot

mmu.resource/MMUResType

mmu.resource/ParseTT

mmu.resource/PermitDMACache

mmu.resource/PFlush

mmu.resource/PFlushA

mmu.resource/PFlushTwo

mmu.resource/PushAll

mmu.resource/PushLine

mmu.resource/PushPage

mmu.resource/PushTwo

mmu.resource/ReadDescriptor

mmu.resource/ReadFastDescriptor

mmu.resource/ReadMMUConfig

mmu.resource/ReadUMFlags

mmu.resource/RootStarter

mmu.resource/SetIndirect

mmu.resource/SetIndirectArray

mmu.resource/WithoutMMU

mmu.resource/WriteDescriptor

mmu.resource/WriteMMUConfig

mmu.resource/--Background--		 	mmu.resource/--Background--

PURPOSE

The mmu.resource is the low-level driver of the mmu.library;

it is just one level above hardware and is the interface of

the mmu.library to the hardware. There is absolutely no need

to make use of this resource whatsoever; the calling syntax is

wierd and requires assembly language necessarely. Many things

within this resource work much different than you might expect.

In case the mmu.resource is found available when the mmu.library

is opened, the library will use the resource to get its dirty

work done. That is, the construction of the MMU table descriptors,

the exception handler and similar. If the mmu.resource is not

found active, the mmu.library tries to figure this out itself.

It will check for the available routine and will use internal

routines that provide similar functions than the resource. It

will not try to build a mmu.resource, though.

The mmu.resource could be used to make use of the mmu.library

in an environment that is not supported by the library initially,

as for example more advanced processors, external MMUs or even

"software emulated" MMUs for emulators. Writing a mmu.resource

for a non-68K MMU might be possible provided the programming

model of this CPU is close enough to that of the Motorola's.

In order to make the mmu.library use your resource routines,

your hardware/software should make the resource available on

boot-up by means of AddResource() before the mmu.library gets

loaded.

The calling syntax of the mmu.resource is, indeed, strange.

This is because most of its functions are used by the core

code of the mmu.library, and hence must expect the a6 register

loaded with the base pointer of the mmu.library, and not this

resource. Some calls require multiple return values in several

registers, or even the condition-code register. About the only

reason why this is so is speed, and tradition - as to mimic the

former calling syntax of the mmu.library internal routines.

Even though the mmu.resource is just a resource, the typical

library type entries LibOpen(), LibClose(), LibExpunge() and

LibExtFunc() should be available. They are currently not used,

though.

mmu.resource/CheckMMUInterface			mmu.resource/CheckMMUInterface

NAME

CheckMMUInterface - check for the required API interface level

SYNOPSIS

okflag = CheckMMUInterface( version, mmubase, mmuresbase );

d0				d0	a0	a6

BOOL CheckMMUInterface( UWORD, struct MMUBase *, struct MMUResBase);

FUNCTION

Checks whether the expected API level of the mmu.resource matches

the requirements of the mmu.library and delivers the mmu.library

base pointer to the resource.

INPUTS

version	- the version number of the mmu.resource API the mmu.library

expects to be available. This need not to coincide with the version of

the mmu.library, though.

mmubase - base pointer of the mmu.library based for internal use of

the resource.

mmuresbase - base pointer of the resource itself.

RETURNS

TRUE in case the resource can service the API whose version is

indicated by the "version" argument; FALSE otherwise.

NOTES

Note that the API version of the mmu.library need not to coincide

with its library version. The lowest available API level of the

library is 43; you should currently check for this value and fail

if you get anything different. Future versions of the mmu.library

might require an enhanced interface and will then bump this

release number.

This function will be called first by the mmu.library as soon as

it detects that the mmu.resource is available. If the result

value is FALSE, the mmu.library will revert to its own internal

CPU/MMU check and will try to select a proper internal set of

support routines.

BUGS

SEE ALSO

mmu.resource/MMUResType					mmu.resource/MMUResType

NAME

MMUResType - provide a one-byte identifier specifying the

MMU type that gets supported

SYNOPSIS

type = MMUResType( mmuresbase );

d0		      a6

char MMUResType( struct MMUResBase * );

FUNCTION

Returns the ID of the mmu that is supported by this resource. This

number will be forwarded to application programs by means of the

mmu.library call GetMMUType().

INPUTS

mmuresbase	-	a pointer to the mmu.resource base.

RETURNS

an identifier specifying the mmu that is available.

NOTES

BUGS

SEE ALSO

mmu.library/GetMMUType()

mmu.resource/AdjustCtxtSettings			mmu.resource/AdjustCtxtSettings

NAME

AdjustCtxtSettings - optimize a MMUContext for best performance

SYNPOSIS

okflag = AdjustCtxtSettings( ctxt, mmubase );

d0		     	      a5     a6

BOOL AdjustCtxtSettings( struct IMMUContext *, struct MMUBase *);

FUNCTION

This function is called after the mmu.library identified the

initial MMU profile by means of ReadMMUConfig(). It is supposed

to modify the read ctxt->ctx_LevelxBits, ctxt->PageBits	and

ctxt->ctx_InitialShift bits in the following way, guaranteeing

that the sum of all these bit counters remains 32 - the number of

bits required to address the full address range of the MC68K

processor.

- the initial shift must be arranged to be zero by possibly

enlarging the number of bits in the levels below.

- the page size should be adjusted to a reasonable default

value if it is supposed to be non-sensical. This depends on

your own choice, but as the mmu.library gets rather un-

effective for small pages, page sizes below 1K and above 8K

should possibly be avoided.

INPUTS

ctxt - a pointer to the IMMUContext structure to be modified.

mmubase - base pointer of the mmu.library.

RESULTS

a TRUE/FALSE indicator telling the mmu.library whether the

resource was successful to establish a useable setting.

NOTES

Note that a6 is loaded with the base of the mmu.library, not

that of the resource.

This function need not to update all the mask values of the

IMMUContext. This is done by the library internally afterwards.

The internal mmu.library drivers do not try to modify the page

size, except for the 68030 and 68851 drivers which will reject

to use a page size smaller than 1K and will then use a default

setting. It is probably a good idea to use the same approach

to come to a useful MMU setup.

BUGS

SEE ALSO

ReadMMUConfig()

mmu.resource/ReadMMUConfig			mmu.resource/ReadMMUConfig

NAME

ReadMMUConfig() - read out the MMU register setup

SYNOPSIS

ReadMMUConfig( config, mmubase );

a0	a6

void ReadMMUConfig(struct MMUConfig *, struct MMUBase *);

FUNCTION

Reads all relevant MMU hardware registers and fills them into

the IMMUConfig structure for further processing.

INPUTS

config - pointer to the struct MMUConfig to fill out.

mmubase - base pointer to the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This call need not to make a complete backup of the MMU setup;

only those registers that are relevant for the MMU programming

model need to be saved. The meaning of the components of the

MMUConfig structure are, even though documented, up to you and

the hardware requirements.

This function is called in supervisor mode and should return

to the caller with an RTS.

BUGS

SEE ALSO

mmu/config.h,WriteMMUConfig(),BuildMMUConfig()

mmu.resource/WriteMMUConfig			mmu.resource/WriteMMUConfig

NAME

WriteMMUConfig() - load an MMU configuration into the hardware

SYNOPSIS

WriteMMUConfig( config, mmubase );

a0	  a6

void WriteMMUConfig(struct MMUConfig *, struct MMUBase *);

FUNCTION

Writes the configuration described by the struct MMUConfig into

the hardware and flushes caches as required.

INPUTS

config - pointer to a struct MMUConfig to be loaded into the

hardware

mmubase - pointer to the mmu.library base.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This call installs all relevant data of the MMUConfig back into

the hardware; dependent on the MMU, various caches must be

flushed here as well. The suggested operation works as follows:

- interrupts should be disabled by writing into the sr

- caches should be pushed back if required

- the MMU should be disabled,

- address translation caches should be flushed,

- transparent translation should be installed,

- MMU root pointers should be installed,

- MMU translation registers should be installed,

- caches should be flushed/pushed again.

This function is called in supervisor mode, it should return to

the caller by means of an RTS.

The function may leave interrupts disabled; they will be re-

enabled as soon as the supervisor mode is left by the caller.

This call need not to install all registers; only the registers

that are relevant to the MMU programming model are required to

be written back.

On a context switch, the mmu.library will compare all components

of the MMUContext except the two (user/supervisor) root pointers.

If these extended registers are equal, a short context switch by

means of LoadRoot() is engaged; otherwise, a complete context

switch by this function will be run.

BUGS

SEE ALSO

mmu/config.h,ReadMMUConfig(),BuildMMUConfig(),LoadRoot()

mmu.resource/BuildMMUConfig			mmu.resource/BuildMMUConfig

NAME

BuildMMUConfig() - build a new MMU configuration from templates

SYNOPSIS

ok = BuildMMUConfig( uctx, sctx, defconfig, destconfig, mmubase);

d0		      a0    a1	    a2		a3	  a6

BOOL BuildMMUConfig(struct IMMUContext *, struct IMMUConctext *,

struct MMUConfig *,struct MMUConfig *,

struct MMUBase *);

FUNCTION

Fills in a new struct MMUConfig by combining data from the default

configuration, the user and the supervisor context. This function

is called by the mmu.library whenever the configuration of a new

context must be created. It should write the MMU root pointers of

the user and supervisor contexts into the MMUConfig, and fill the

remaining data of the configuration with default values from the

default configuration.

INPUTS

uctx -	the user context whose configuration should be build. This

shall be the origin of the user root pointer. Further, the

page layout shall be taken from the uctx->ctx_LevelABits...

uctx->ctx_LevelDBits, uctx->ctx_PageBits bits and should

modify the translation control provided by defconfig ac-

cordingly before this data goes into the translation control

of destconfig.

Further, this procedure should clear transparent translation

if the uctx->ctx_Flags say so.

sctx -	the supervisor context of the uctx. This is the origin of the

supervisor root pointer.

defconfig - the default MMU configuration that was read by the

mmu.library on startup. Remaining registers of the MMU

programming model should be taken from here.

destconfig - destination MMU configuration to fill out.

mmubase - pointer to the mmu.library base.

RESULTS

a boolean success/failure indicator. Should return TRUE in case

the configuration could be build successfully, FALSE otherwise

if for example the page configuration specified by the context is

not available.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

BUGS

SEE ALSO

mmu/config.h,ReadMMUConfig(),WriteMMUConfig()

mmu.resource/LoadRoot					mmu.resource/LoadRoot

NAME

LoadRoot() - perform a minor context switch

SYNOPSIS

LoadRoot( uroot, sroot, mmubase);

a0	   a1	  a6

void LoadRoot( void *, void *, struct MMUBase *);

FUNCTION

Performs a minor context switch by exchanging user, supervisor or

both MMU root pointers.

INPUTS

uroot - the user MMU root pointer or NULL in case the user root

pointer shall remain unaltered.

sroot -	the supervisor MMU root pointer or NULL in case it need not

to be touched.

mmubase - pointer to the mmu.library base.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This function installs either a new user or supervisor root pointer

into the MMU hardware; therefore, this function is called within

supervisor mode and shall return with an RTS to the user.

This function shall perform the following operations:

- flush the address translation cache,

- flush/push the cache if the cache is logically indexed. A

physically indexed cache need not to be pushed.

- load the user root pointer with the new value

- flush old user root pointer from a RTC, if available,

and old user root<>new user root

- load the supervisor root pointer with the new value,

- flush old supervisor pointer from a RTC, if available,

and old supervisor root<>new supervisor root

- flush the ATC again

The mmu.library calls either this function or WriteMMUConfig()

on a context switch. If the MMU registers of old and new context

are identically except for user and supervisor root pointer,

a minor context switch is run by this function. Otherwise, a

major context switch by means of WriteMMUConfig() is performed.

BUGS

SEE ALSO

WriteMMUConfig()

mmu.resource/DefineMMUContext			mmu.resource/DefineMMUContext

NAME

DefineMMUContext() - fill IMMUContext components with MMU specific

data from a MMUConfig.

SYNOPSIS

DefineMMUContext( config, ctxt, mmubase);

a0	   a5	   a6

void DefineMMUConfig( struct MMUConfig *, struct IMMUContext *,

struct MMUBase *);

FUNCTION

Reads data from the MMUConfig structure, interprets the page

layout defined by this data and places the results in the

IMMUContext pointed to by ctxt. This function shall fill

in:

ctxt->ctx_InitialShift	with the initial shift value

ctxt->ctx_LevelABits	with the number of address bits to

address level A of the MMU table.

ctxt->ctx_LevelBBits

ctxt->ctx_LevelCBits

ctxt->ctx_LevelDBits	ditto up to level D

ctxt->ctx_PageBits	number of bits of a logical address

that define the offset within a page.

If the MMU is disabled, the above should be set to reasonable

defaults, as for example 4K pages.

ctxt->ctx_LevelAAlign	Page alignment al� AllocAligned() that

is required for the level A descriptor

level. This must be a power of two.

ctxt->ctx_LevelBAlign

ctxt->ctx_LevelCAlign

ctxt->ctx_LevelDAlign	ditto for levels B..D

ctxt->ctx_DescriptorAlign	alignment restrictions for

indirect descriptors.

ctxt->ctx_PageMemMask	binary mask of all mapping properties that

shall be touched when allocating memory for

the MMU table.

ctxt->ctx_PageMemProp	binary mask of page properties that will be

enabled/disabled for memory allocated for MMU

page descriptors.

The above two are only kept care of if the user specified

DescriptorCacheInhibit ON

in the ENVARC:MMU-Configuration file. Then, the properties of mmu

page memory are filtered by

newprops = oldprops & (~PageMemMask) | PageMemProps

to get the new property flags for the MMU table memory. Otherwise,

these two are ignored.

ctxt->ctx_PageMemForbid	property flags that must not be set for memory

that got allocated for the MMU table.

If the mmu.library finds one of the above properties set for memory

that it allocated, it will guru. Typically, this mask should contain:

MAPP_BLANK, MAPP_INVALID, MAPP_SWAPPED, MAPP_TRANSLATED, MAPP_ROM,

MAPP_WRITEPROTECTED, MAPP_BUNDLED, MAPP_SUPERVISORONLY, MAPP_INDIRECT

Note that memory allocated by MEMF_PUBLIC should never have any of the

properties above. Note further that the mmu.library is able to work

with MAPP_REMAPPED memory for mmu descriptors since it will first

translate the physical address to logical before using it for the

descriptors. This requires, though, that the memory is always

accessable under its physical address as well.

ctxt->ctx_LargestPageSize	largest possible page size with

this MMU; not necessarely the

active page size.

This value is required by the mmu.library to compute the

RemapSize() value. It should be a power of two.

INPUTS

config - struct MMUConfig to be read out.

ctxt - struct IMMUContext to be filled in.

mmubase - library base of the mmu.library.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is called by the mmu.library whenever a new context must be

build; you need not to touch any other mask/flags fields of the

IMMUContext as the library will compute them itself from the

data you provided. Note further that you should not cheat here

with the settings to get a more reasonable setup. The mmu.library

will later on call AdjustCtxtSetting() on this context to give

you the chance to make some adjustments on the MMU setup.

DefineMMUContext( config, ctxt, mmubase);

a0	   a5	   a6

void DefineMMUConfig( struct MMUConfig *, struct IMMUContext *,

struct MMUBase *);

BUGS

SEE ALSO

AdjustCtxtSetting(), mmu/RemapSize(), mmu/context.h,

mmu/config.h

mmu.resource/AdjustMMUContext			mmu.resource/AdjustMMUContext

NAME

AdjustMMUContext() - integrate user supplied page layout settings into

an IMMUContext

SYNOPSIS

errcode = AdjustMMUContext( depth, destctxt, parent , mmubase );

d0			     d0 	a0	a1	a6

LONG AdjustMMUContext( LONG depth, struct MMUContext *,

struct IMMUContext *, struct IMMUBase *);

FUNCTION

This function gets called on CreateMMUContextA() to integrate

the user supplied page model into the context, given a parent/

template context to take default settings from. This function

shall try to find a possible page configuration fit for the user

supplied data within the destctxt, or shall fail if it is not

possible to generate the user defined page layout.

INPUTS

depth - depth of the MMU tree in levels, or negative if not

specified by the user. Then, this function shall use

the default depth from the "parent" context.

destctxt - destination context to generate a page layout for;

specifically, the destctxt->ctx_LevelABits to

destctxt->ctx_PageBits must be defined. Initially,

these are either filled with the user-specified values,

or 0xff for "default values". It is the job of this

function to fill all the components of the IMMUContext

that are set to 0xff such that the page layout is

supported by the hardware and as close as possible to

that of the parent. Ideally, the parent configuration

should be used to allow minor context switches between

the parent and this context. If possible, at least the

page size of parent and destctxt should be made identically,

unless the user tries to setup its own page size.

parent - struct IMMUContext to take default settings from.

mmubase - library base pointer of the mmu.library

RESULTS

An error code or zero on success. The following error codes shall

be used:

CCERR_INVALID_PARAMETERS	the parameters make no sense, as

for example the sum of all level and

page bits is not 32.

CCERR_UNSUPPORTED		the parameters are consistent, but not

supported by the MMU hardware.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

BUGS

SEE ALSO

mmu/context.h, mmu/CreateMMUContextA()

mmu.resource/ParseTT					mmu.resource/ParseTT

NAME

ParseTT() - integrate the transparent translation registers into

the MMU table layout.

SYNOPSIS

ok = ParseTT( ctxt, config, mmubase);

d0		a0	a1	a6

BOOL ParseTT( struct IMMUContext *, struct MMUConfig *, struct MMUBase *);

FUNCTION

Parses the transparent translation registers given by the MMUConfig

and integrates the definitions made by the TTx registers into the

MMU setup defined by the ctxt. The attempt of this function is to

get rid of the TTx definitions and mimic their function by placing

similar page definition data into the MMU table of the context.

INPUTS

ctxt -	the IMMUContext into which the TTx registers shall be integrated.

Especially, the following flags in ctxt->ctx_Flags should be

considered:

CTXF_CLEARDTT0		Is set on entry if the DTT0 register

shall be ignored.

This flag gets set by the mmu.library on startup if the

ENVARC:MMU-Configuration contains the "ClearTTx" command.

On exit, this flag shall be set in case the DTT0 setup could

be successfully integrated into the MMU table of ctxt.

CTXF_CLEARDTT1, CTXF_CLEARITT0, CTXF_CLEARITT1

similar.

config - struct MMUConfig containing the current setup of the TTx

registers that are about to be integrated.

mmubase - pointer to the mmu.library base

RESULTS

a boolean success/failure indicator.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This function might require the help of two internal mmu.library

functions that are hereby documented purely for the use of the

mmu.resource:

ok = AddTransparentTranslation( ctxt, amask, props, propmask );

d0				 a5	a1     d0	d1

BOOL AddTransparentTranslation( struct IMMUContext *, UWORD,

ULONG, ULONG );

ok = OrTransparentTranslation( ctxt, amask, props, propmask );

BOOL OrTransparentTranslation(	struct IMMUContext *, UWORD,

ULONG, ULONG );

Both functions work very much the like with the difference that

the first attempts to insert all TTx properties into the context,

whereas the second uses the most conservative setting when the

old context definition and the TTx definition conflict.

Arguments of these calls are as follows:

ctxt	-	MMUContext to integrate TTx settings into

amask	-	an address and address mask. The lower 8 bits

of amask define a mask, the upper 8 an address.

The address mask is interpreted as bits 31..24 of a full mask,

where a one-bit indicates that the corresponding address bit is

ignored on address comparison. Bits 23..0 are always understood

to be one, i.e. are always ignored. Hence, TTx registers are

limited to define 16MB chunks of memory at once.

The address bits define bits 31..24 of an address to compare

against.

The code will, therefore, perform the following operation:

if (((address>>24)^(amask >> 8)) & (~amask & 0xff) == 0) {

/* modify context */

}

If the above comparison is true, the mmu.library will modify

the 16MB segment starting at "address" by means of the propmask

and the props arguments. "props" defines the new values of the

property flags, "propmask" which property bits are to be

altered.

BUGS

SEE ALSO

mmu/context.h

mmu.resource/RootStarter			      mmu.resource/RootStarter

NAME

RootStarter - get a pointer to the MMU root as abstract table

descriptor

SYNOPSIS

ok = RootStarter( config, atd, type, mmubase);

d0		    a1	   a0	d0   a6

LONG RootStarter(struct MMUConfig *,struct AbstractDescriptor *,

UBYTE, struct MMUBase *);

FUNCTION

Fills in the "AbstractDesctriptor" describing the location of the

MMU root table pointer. adt->adt_Pointer should be filled with

the address(!) of a memory location containing the user or

superivor root table pointer. This is typically the address of

one of the root pointer components within the MMUConfig passed

in.

INPUTS

config	- struct MMUConfig containing the user and supervisor

root pointers.

atd -	  AbstractDescriptor to be filled in. The following

fields must be provided:

atd->atd_Pointer	pointer to the root pointer

atd->atd_LowerLimit	set to zero

atd->atd_UpperLimit	set to zero

atd->atd_Type		set to ATDT_TABLE for short

descriptors (four bytes),

ATDT_TABLE|ATDT_LONG for long

descriptors (eight bytes)

type -	0 if you need to provide the user root pointer, non-

zero otherwise. In case the MMU programming model described

by "config" does not distinguish between user and supervisor

access, ignore this type.

mmubase - pointer to the mmu.library base.

RESULTS

ok should be set to zero in case the MMU root could not be read.

In this case, the resource will have caused a guru already.

The result code shall be -1 in case the MMU is enabled and the

root could be read, and +1 in case the MMU is disabled. The

atd need not to be filled out in this case.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This function is used by the mmu.library to initiate the mmu table

scan of the active MMU configuration.

BUGS

SEE ALSO

ReadDescriptor(), mmu/config.h, mmu/descriptor.h

mmu.resource/ReadDescriptor			mmu.resource/ReadDescriptor

NAME

ReadDescriptor() - read and interpret an hardware MMU descriptor

SYNOPSIS

ReadDescriptor( deptr, atd, type, level, ctxt, mmubase );

a1    a0    d0    d1	 a5	a6

ReadDescriptor( ULONG *, struct AbstractDescriptor *, UBYTE ,

UBYTE, struct IMMUContext *, struct MMUBase *);

FUNCTION

Reads and interprets an MMU table/page descriptor pointed to by

deptr, and fills the abstraction data into the atd.

INPUTS

deptr -	points to the descriptor to read.

atd -	the AbstractDescriptor to be filled in.

type -	type of the descriptor to be read. This has the bit

ATDB_LONG set for long descriptors. All other bits

shall be ignored.

level -	MMU table level to read the descriptor from. Zero

identifies level A, -1 is used in case this function

shall read and interpret the root pointer itself, as

supplied by RootStarter(). "deptr" points then to a

memory location containing(!) the root pointer.

ctxt -	struct IMMUContext this parsing is done within. The

IMMUContext provides all bitmasks to isolate property

bits and pointers from the descriptor, i.e. in

ctxt->ctx_LevelAMask...ctxt->ctx_PageMask. Note that

the LevelAMask is required for reading pointers that

point to a level A table, i.e. the root pointer.

mmubase - pointer to the mmu.library base.

The AbstractDescriptor must be filled in as follows:

atd->atd_Pointer	pointer to the next MMU descriptor table

or the physical page address or the

indirect descriptor.

atd->atd_Properties	properties described by this descriptor.

atd->atd_LowerLimit	in case this is a long descriptor, the

lower page limit in case this descriptor

uses a lower limit, or zero otherwise.

atd->atd_UpperLimit	in case this descriptor is long and uses

an upper page limit, fill this limit into

here without the "upper limit signal bit".

Otherwise, set to 0x7fff.

atd->atd_ThisType	type of this descriptor. The following bits

are available to signal the type:

ATDB_TABLE	is a table descriptor

ATDB_INDIRECT	is an indirect descriptor

ATDB_PAGE	is a page descriptor

(includes early termination descriptors)

ATDB_INVALID	is an invalid descriptor

(set properties to MAPP_INVALID, too!)

atd->atd_NextType	type of the descriptors this descriptor

points to if this is a table or an invalid

descriptor. The following bits should be

set:

ATDB_LONG	Descriptors in the table this descriptor

points to are of long type (eight bytes)

ATDB_TABLE	This descriptor points to a table of new

descriptors, possibly including page,

indirect or invalid descriptors.

ATDT_INVALID	This descriptor doesn't point to a next

descriptor because it is either a page

descriptor, an early termination descriptor

or invalid.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This function is used by the mmu.library to scan an already loaded

MMU table.

BUGS

SEE ALSO

RootStarter(), mmu/descriptor.h

mmu.resource/ReadFastDescriptor			mmu.resource/ReadFastDescriptor

NAME

ReadFastDescriptor() - read an MMU descriptor quickly.

SYNOPSIS

ReadFastDescriptor( deptr, atd, type, level, ctxt, mmubase );

a1    a0    d0    d1	 a5	a6

ReadFastDescriptor( ULONG *, struct AbstractDescriptor *, UBYTE ,

UBYTE, struct IMMUContext *, struct MMUBase *);

FUNCTION

Reads and interprets an MMU table/page descriptor pointed to by

deptr, and fills the abstraction data into the atd.

This is similar to ReadDescriptor except that this function is

allowed to take some shortcuts. It is not assumed to fill in

any property flags or atd->atd_NextType, neither need it work

with long descriptors.

INPUTS

deptr -	points to the descriptor to read.

atd -	the AbstractDescriptor to be filled in.

type -	type of the descriptor to be read. This has the bit

ATDB_LONG set for long descriptors. All other bits

shall be ignored.

level -	MMU table level to read the descriptor from. Zero

identifies level A, -1 is used in case this function

shall read and interpret the root pointer itself, as

supplied by RootStarter(). "deptr" points then to a

memory location containing(!) the root pointer.

ctxt -	struct IMMUContext this parsing is done within. The

IMMUContext provides all bitmasks to isolate property

bits and pointers from the descriptor, i.e. in

ctxt->ctx_LevelAMask...ctxt->ctx_PageMask. Note that

the LevelAMask is required for reading pointers that

point to a level A table, i.e. the root pointer.

mmubase - pointer to the mmu.library base.

See ReadDescriptor() for further details.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This streamlined version is used by the mmu.library as soon as

it knows that it works on tables it build itself.

BUGS

SEE ALSO

ReadDescriptor(), mmu/descriptor.h

mmu.resource/CmpDescriptor			mmu.resource/CmpDescriptor

NAME

CmpDescriptor() - compare two descriptors by an offset

SYNOPSIS

iseq = CmpDescriptor( descr1 , descr2 , type, delta, mmubase);

ccr			a0	a1	 d0	d1	a6

void CmpDescriptor( ULONG *, ULONG *, UBYTE, ULONG, struct MMUBase *);

FUNCTION

Compares whether descriptor one is equal to the descriptor up to an

address displacement delta.

Hence, if both descriptors are page descriptors, the call checks

whether the destination page of descr1 equals the destination page

of descr2 minus delta, and whether the properties are identically up

to the MAPP_USED and MAPP_MODIFIED bits.

If both descriptors are indirect, the call checks whether they point

both to the same target descriptor.

If both descriptors are invalid, they are always considered equal.

INPUTS

descr1	- the base descriptor to compare against

descr2	- the descriptor that it is compared with.

type	- has ATDT_LONG set for long descriptors, otherwise

descr1 and descr2 point to short descriptors.

has ATDT_INDIRECT set for indirect descriptors.

delta   - address displacement of descr2 relative to descr1.

mmubase - base address of the mmu.library

RESULTS

This call returns its result in the condition code register and is

hence not callable from C; note that it is not supposed to be called

by anything but the mmu.library anyhow.

It sets the condition code register Z flag if the descriptors are

considered to be equal, otherwise Z is cleared.

Register a0 must not be touched by this call, and register a1 must

be advanced by one descriptor, i.e. either four or eight bytes,

depending on whether the descriptor is long or short.

This strange syntax was chosen to make the call as fast as possible

and hence to speed up the MMU table scanner.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This call is used by the mmu.library internal MMU table parser to

detect continously mapped memory blocks. It shall execute as fast

as possible.

BUGS

The calling syntax is ugly.

SEE ALSO

ReadDescriptor()

mmu.resource/IncrementDescriptor	    mmu.resource/IncrementDescriptor

NAME

IncrementDescriptor() - advance a pointer into an MMU descriptor table

and provide an increment

SYNOPSIS

increment = IncrementDescriptor( atd, index, mmubase );

d0				  a0	d0	a6

ULONG IncrementDescriptor( struct AbstractDescriptor *, ULONG,

struct MMUBase *);

FUNCTION

Checks atd->atd_NextType and returns the increment required to

get to the next descriptor; further, loads a pointer at the

descriptor within the descriptor table pointed to by atd->atd_Pointer.

INPUTS

atd -	an abstract descriptor describing the start of the descriptor

table and the type of descriptors within this table.

atd->atd_NextType is used to check whether short or long

descriptors are found within this table,

atd->atd_Pointer is assumed to be the base pointer to a

descriptor table of descriptors of type atd->atd_NextType.

index -	the index of the descriptor within the table pointed to by

atd->atd_Pointer to provide as secondary output.

mmubase - pointer to the mmu.library base.

RESULTS

increment - the byte increment to get from one descriptor within the

table atd->atd_Pointer points to to the next one. This

is typically four for short and eight for long descriptors.

a0 contains a secondary result that is not useable from C (but note

that this function must not be called by anything but the mmu.library

anyhow), namely the address of the descriptor within the table at

the table entry given by index.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This call is used by the mmu.library internal MMU table parser to

advance by a table descriptor and to pick a descriptor from a

table.

BUGS

The calling convention is ugly.

SEE ALSO

CmpDescriptor()

mmu.resource/WriteDescriptor			mmu.resource/WriteDescriptor

NAME

WriteDescriptor() - build a hardware MMU table descriptor from an

abstract table descriptor.

SYNOPSIS

descr = WriteDescriptor( atd, level, ctxt, mmubase );

d0			 a0	d1    a5     a6

ULONG WriteDescriptor(	struct AbstractDescriptor *a0, WORD level,

struct IMMUContext *, struct MMUBase *);

FUNCTION

Builds a hardware MMU table descriptor from an abstract description

for a given level of the MMU tree.

INPUTS

atd -	AbstractDescriptor providing the information concerning the

descriptor type to build. The following components are

relevant:

atd->atd_Pointer:	Either the address of the next

descriptor table for table

descriptors, or the physical address

for page descriptors, or the address

of the destination descriptor for

indirect descriptors, or the

descriptor contents for invalid

descriptors; in the last case, several

bits of the pointer will be masked

out, as they are required to signal an

indirect descriptor type.

atd->atd_Properties:	Properties of the descriptor to be

build. MAPP_INVALID or MAPP_SWAPPED

signals that an	invalid descriptor

shall be build, MAPP_INDIRECT signals

an indirect descriptor.

atd->atd_LowerLimit:	ignored, only short descriptors are

supported

atd->atd_UpperLimit:	ignored for the same reason.

atd->atd_ThisType:	Descriptor type to build. This must

have the ATDB_LONG bit cleared. The

following types are supported:

ATDT_INVALID	Build an invalid descriptor, as if

MAPP_INVALID or MAPP_SWAPPED has been

set. Descriptor contents is within

atd->atd_Pointer.

ATDT_PAGE	Build a page descriptor or an early

termination page descriptor. This

reverts to indirect or invalid de-

scriptors if the corresponding

property flags are set. The physical

page address is in atd->atd_Pointer.

In case MAPP_INVALID or MAPP_SWAPPED

are set, atd_Pointer contains the

data for the invalid descriptor.

In case MAPP_INDIRECT is set, the

atd_Pointer contains the address of

the target descriptor.

ATDT_TABLE	Build a table descriptor. atd_Pointer

points to the next descriptor table.

ATDT_INDIRECT	This value is not allowed. Instead,

indirect descriptors must be build by

MAPP_PAGE and properties of

MAPP_INDIRECT.

atd->atd_NextType	ingored.

level - level at which the descriptor shall be build. Level = 0

indicates level A of the MMU tree, level = -1 indicates that

a root descriptor shall be build.

ctxt - 	pointer to the IMMUContext the descriptor will be part of.

mmubase - pointer to the MMU library base.

RESULTS

descr - the generated descriptor. Note that 0L need not to indicate

an error.

Error conditions are delivered by means of the Z bit of the CCR

register. If the Z bit is one, the requested descriptor could not be

build. This happens for example if a descriptor of type ATDT_PAGE

shall be build at non-page level and the MMU does not support early

termination descriptors. If everything is fine, the Z bit will be

cleared.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

Note that the usage of the abstract descriptor is slightly different

from that of ReadDescriptor(). ReadDescriptor() delivers a descriptor

type of ATDT_INDIRECT for indirect descriptors, whereas

WriteDescriptor() expects ATDT_PAGE with properties set to

MAPP_INDIRECT to build this descriptor type.

This call is used by the mmu.library table build algorithm to generate

new descriptors.

BUGS

The name is misleading, GenerateDescriptor() would fit better as

this function never writes any descriptor out to memory.

The calling syntax is non-standard and ugly.

SEE ALSO

ReadDescriptor(), mmu/descriptor.h

mmu.resource/FindDescriptor			mmu.resource/FindDescriptor

NAME

FindDescriptor() - find an MMU descriptor by logical address.

SYNOPSIS

level = FindDescriptor( addr, ctxt , mmubase );

d0			 a0     a5	a6

WORD FindDescriptor( ULONG, struct IMMUContext *, struct MMUBase *);

FUNCTION

This function scans the MMU tables to detect the descriptor that

is responsible for translating the logical address to a physical

address. This descriptor address is returned as secondary result

code in register a0.

For indirect descriptors, this function returns the address

of the descriptor pointing to the target descriptor, and not the

descriptor itself.

The primary result code is the level at which the descriptor

was found. Zero indicates a descriptor at level A of the MMU

table.

INPUTS

addr - logical address whose descriptor shall be found.

ctxt - IMMUContext managing this descriptor.

mmubase - pointer to the mmu.library base.

RESULTS

level - page level the descriptor has been found at. This need

not to be the page level as invalid or early termination

descriptor might abort a table search earlier.

A secondary result is returned in register a0; this is the address

of the descriptor translating the logical address; for indirect

descriptors, this is not the final target descriptor, but the

indirect descriptor pointing to the final target.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

This call has to read MMU descriptors for obvious reasons. As the

mmu.library does not necessarely place these in non-cacheable memory,

your code must keep care to push the descriptors back to memory as

soon as they have been read. Not following this advice may either

cause inconsistent MMU mapping or - even worse - hangs of the MMU.

BUGS

The calling syntax is ugly.

SEE ALSO

mmu.resource/PFlush					mmu.resource/PFlush

NAME

PFLush()	-	flush ATC entries for a given logical address

SYNOPSIS

PFlush( addr, mmubase );

a0    a6

void PFlush( ULONG, struct MMUBase * );

FUNCTION

Flushes the ATC entry for the descriptor describing the mapping

of the passed in logical address.

INPUTS

addr - logical address whose descriptor needs to be flushed.

mmubase - base pointer of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

Since the mmu.library enforces a separate user/supervisor model,

this call must flush the ATC entries for both, user and supervisor

tree.

Note further that early termination descriptors take up more than

one ATC entry. Hence, this call flushes a descriptor for one page,

but not necessarely for all pages it describes.

BUGS

SEE ALSO

PFlushA(), PFlushTwo()

mmu.resource/PFlushA					mmu.resource/PFlushA

NAME

PFlushA()	-	flush the address translation cache completely

SYNOPSIS

PFlushA( mmubase );

a6

void PFlushA( struct MMUBase *);

FUNCTION

Flushes the address translation cache of the MMU completely.

INPUTS

mmubase - library base pointer of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

BUGS

SEE ALSO

PFlush(), PFlushTwo()

mmu.resource/PFlushTwo					mmu.resource/PFlushTwo

NAME

PFlushTwo()	-	flush two descriptors from the ATC

SYNOPSIS

PFlushTwo( addr1, addr2, mmubase);

a2    a3	a6

void PFlushTwo( ULONG, ULONG, struct MMUBase * );

a2	 a3		a6

FUNCTION

Flushes two descriptors from the MMU address translation cache by

the logical addresses they describe.

INPUTS

addr1	-	first logical address whose descriptor shall be

flushed,

addr2	-	second logical address to be flushed

mmubase	-	base pointer to the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

Since the mmu.library enforces a separate user/supervisor model,

this call must flush the ATC entries for both, user and supervisor

tree.

Note further that early termination descriptors take up more than

one ATC entry. Hence, this call flushes a descriptor for one page,

but not necessarely for all pages it describes.

This call does not flush an address range, but really only two

descriptors; this call is used to speed-up the CachePreDMA() and

CachePostDMA() function patches. Otherwise, it is completely

equivalent to calling PFlush() twice with addr1 and addr2 as

arguments, respectively.

BUGS

SEE ALSO

PFlush(), PFlushA()

mmu.resource/PushLine					mmu.resource/PushLine

NAME

PushLine()	-	push a cache line back to memory

SYNOPSIS

PushLine( addr, mmubase );

a0	  a6

void PushLine( ULONG, struct MMUBase * );

FUNCTION

Pushes the cache line containing the logical address *a0 back

to memory. Hence, the sixteen-byte block containing the passed

in address will be written out from the cache into memory, if

the corresponding cache line is dirty. The cache line is then

marked as "flushed" and non-valid.

INPUTS

addr - 	the logical address contained in the cache line to be

pushed.

mmubase - base pointer to the mmu.library base.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

Depending on the cache, this call might require to push more than

just a line; it might also flush the cache completely if this is

more efficient. This call pushes the data cache, and flushes the

instruction cache; hence, it operates on all available caches.

BUGS

SEE ALSO

PushPage(), PushAll(), PushTwo()

mmu.resource/PushPage					mmu.resource/PushPage

NAME

PushPage()	-	push a MMU page back to memory

SYNOPSIS

PushPage( addr, mmubase );

a0	  a6

void PushPage( ULONG, struct MMUBase * );

FUNCTION

Pushes the page containing the logical address *a0 back to

memory. Hence, the memory page containing the passed in address

will be written out from the cache into memory, if the

corresponding cache line is dirty. The cache lines are then marked

as "flushed" and non-valid.

INPUTS

addr - 	the logical address contained in the page to be	pushed.

mmubase - base pointer to the mmu.library base.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

Depending on the cache, this call might require to push more than

just a page; it might also flush the cache completely if this is

more efficient. This call pushes the data cache, and flushes the

instruction cache; hence, it operates on all available caches.

BUGS

Due to a hardware bug of the 68060, this call might not	flush

the instruction cache as indicated if the address is not

aligned to a page boundary. Hence, you are adviced to align

the address before calling this as the mmu.resource does not

try to work around this bug.

SEE ALSO

PushLine(), PushAll(), PushTwo()

mmu.resource/PushAll					mmu.resource/PushAll

NAME

PushAll()	-	push and flush all caches completely

SYNOPSIS

PushAll( mmubase );

a6

void PushAll( struct MMUBase * );

FUNCTION

Pushes the data cache completely back to memory, and flushes

the data and instruction cache completely.

INPUTS

mmubase - base pointer to the mmu.library base.

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

BUGS

SEE ALSO

PushLine(), PushPage(), PushTwo(), exec/CacheClearU()

mmu.resource/PushTwo					mmu.resource/PushTwo

NAME

PushTwo()	-	push two pages of the data cache

SYNOPSIS

PushTwo( addr1, addr2, mmubase );

a2	 a3	 a6

void PushTwo( ULONG, ULONG, struct MMUBase * );

FUNCTION

Pushes the data cache entries for the pages containing the indicated

logical addresses from out of cache.

INPUTS

addr1 -	first logical address contained in the first page to be

pushed,

addr2 -	second logical address of the page to be pushed

mmubase - base pointer to the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

This call does not touch the instruction cache; its purpose is to

prepare the edges of a memory block that is about to the transfered

by DMA. This is called as part of the CachePreDMA() resp.

CachePostDMA() functions. Note that it doesn't push a range of pages,

but really only to pages.

BUGS

SEE ALSO

PushLine(), PushPage(), PushAll()

mmu.resource/ForbidDMACache			mmu.resource/ForbidDMACache

NAME

ForbidDMACache() - forbid caching of a page temporarly as long as an

DMA transfer is running.

SYNOPSIS

ForbidDMACache( addr, ctxt, mmubase );

a0    a5     a6

void ForbidDMACache( ULONG, struct IMMUContext *, struct MMUBase *);

FUNCTION

Increments the DMA cache disable counter for the indicated page and

sets the cache mode of the affected page to MAPP_WRITETRHOUGH until

the matching PermitDMACache() is called. This call nests. Further,

this call must be aware of the user selected page attributes.

INPUTS

addr -	a logical address contained in the page to be touched by a

DMA transfer,

ctxt -	a struct MMUContext describing the context the corresponding

MMU tree is relative to.

mmubase - library base of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode. This will be called with all interrupts disabled.

This call must:

- Find the page descriptor for the selected page,

- push it back to memory in case someone read it (illegally, by-

passing the mmu.library functions)

- check the DMA disable counter for the corresponding page.

The mmu.library generates the required counters for you in the

following way: A page-level descriptor table of "N" entries

generates four tables: Table one at offset zero contains the

MMU hardware descriptor, table two at offset "N" contains the

page properties the user selected for the page, or the

"userdata" for invalid or swapped pages, table three at offset

"2*N" contains the DMA disable counters. Table four is still un-

used.

Hence, this call must find the page descriptor of the indicated

page and the size of the page descriptor table. The long word

at byte offset "tablesize * 8" from the page descriptor is the

DMA activity counter, the long word at "tablesize * 4" the

user data or the mapping properties.

- if the DMA disable counter is still zero, and we found a page

descriptor, disable the copyback flag if caching is enabled,

- write the descriptor back to memory,

- push the cache line containing the descriptor to ensure the

MMU will find it.

Note that this call will not work for early termination page

descriptors, invalid, swapped or indirect descriptors. It need

not to work for early termination since the only MMUs that implement

them are the 68851 and 68030 whose CPUs do not offer a copyback

cache and hence do not need this support function. Neither need it

to support invalid or swapped pages since they cannot contain

vital data for a DMA process; it does not support indirect

descriptors since it wouldn't know how to reconstruct them if DMA

is done. This is a documented side condition of DMA and indirect

descriptors that must be kept care of. This function must not crash

if these conditions are not met; it shall not touch the descriptors

then, though.

Note further that the context you get passed in need not to be

the active context; in fact, this function is called with every

available context as soon as a DMA operation is initiated by means

of CachePreDMA() to ensure proper operation even on a context switch

while DMA is active.

This function does nothing in case the MMU does not support copyback,

i.e. for the 68030 and 68851.

BUGS

SEE ALSO

PermitDMACache(), exec/CachePreDMA()

mmu.resource/PermitDMACache			mmu.resource/PermitDMACache

NAME

PermitDMACache() - re-enable caching of a page if DMA done

SYNOPSIS

PermitDMACache( addr, ctxt, mmubase );

a0    a5     a6

void PermitDMACache( ULONG, struct IMMUContext *, struct MMUBase *);

FUNCTION

Decrements the DMA cache disable counter for the indicated page and

restores the previously active caching mode if it reaches zero.

This call nests. Further, this call must be aware of the user

selected page attributes.

INPUTS

addr -	a logical address contained in the page that was touched by a

DMA transfer,

ctxt -	a struct MMUContext describing the context the corresponding

MMU tree is relative to.

mmubase - library base of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode. This will be called with all interrupts disabled.

This call must:

- Find the page descriptor for the selected page,

- check the DMA disable counter for the corresponding page.

The mmu.library generates the required counters for you in the

following way: A page-level descriptor table of "N" entries

generates four tables: Table one at offset zero contains the

MMU hardware descriptor, table two at offset "N" contains the

page properties the user selected for the page, or the

"userdata" for invalid or swapped pages, table three at offset

"2*N" contains the DMA disable counters. Table four is still un-

used.

Hence, this call must find the page descriptor of the indicated

page and the size of the page descriptor table. The long word

at byte offset "tablesize * 8" from the page descriptor is the

DMA activity counter, the long word at "tablesize * 4" the

user data or the mapping properties.

- decrement the DMA disable counter; if it underruns, it shall

be set back to zero to support faulty devices that report a

DMA-done more than once.

- if the DMA disable counter is zero, proceed as follows:

- push the page descriptor back to memory in case someone touched

it, bypassing the documented mmu.library functions,

- check whether this descriptor is really a page descriptor or

invalid or indirect,

- if it is, check whether the user property flags at offset

"tablesize * 4" indicate copyback caching,

- if so, re-enable the copyback cache.

- push the descriptor back to memory so the MMU will find the

proper descriptor

Note that this call will not work for early termination page

descriptors, invalid, swapped or indirect descriptors. It need

not to work for early termination since the only MMUs that implement

them are the 68851 and 68030 whose CPUs do not offer a copyback

cache and hence do not need this support function. Neither need it

to support invalid or swapped pages since they cannot contain

vital data for a DMA process; it does not support indirect

descriptors since it wouldn't know how to reconstruct them if DMA

is done. This is a documented side condition of DMA and indirect

descriptors that must be kept care of. This function must not crash

if these conditions are not met; it shall not touch the descriptors

then, though.

Note further that the context you get passed in need not to be

the active context; in fact, this function is called with every

available context as soon as a DMA operation is terminated by means

of CachePostDMA() to ensure proper operation even on a context switch

while DMA is active.

This function does nothing in case the MMU does not support copyback,

i.e. for the 68030 and 68851.

BUGS

SEE ALSO

ForbidDMACache(), exec/CachePostDMA()

mmu.resource/InstallPage			mmu.resource/InstallPage

NAME

InstallPage() - re-install a page descriptor into the MMU tree

SYNOPSIS

InstallPage( level, tablesize, dest, descr, data, addr, ctxt, mmubase);

d2	d1	a2    d0     d4    a0    a5    a6

void InstallPage( WORD, ULONG, ULONG *, ULONG, ULONG, ULONG,

struct IMMUContext *, struct MMUBase *);

FUNCTION

Re-install a page descriptor into a MMU tree; keep care about DMA

disable counters and carry the used/modified bits over from the last

descriptor. Install or modify page data, adjust the context.

INPUTS

level	-	level of the MMU tree where the descriptor shall be

installed. Zero is level A.

tablesize -	size of the descriptor table at this level in entries

dest	-	physical address where to install the descriptor to

descr	-	the descriptor to install

data	-	secondary page data to install

addr	-	logical address that is described by this descriptor

ctxt	-	struct IMMUContext this descriptor is part of

mmubase	-	library base pointer of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode. This will be called with all interrupts disabled.

This call must:

- check whether we need to install at page level; if so, DMA disable

counters are available at byte offset "tablesize * 8" from the

descriptor destination address "dest".

- if the DMA counter is found active, the descriptor is found to be

a valid page descriptor with copyback enabled, copyback must be

disabled. This is required for proper support of ForbidDMACache()

as the user may want to install a descriptor while a DMA operation

is running on the same page.

- secondary page data must be written to byte offset "tablesize * 4"

from "dest". This page data consists either of the "property flags"

if the page descriptor is valid or indirect, or on the "user data"

for invalid or swapped non-repairable pages.

- check again whether the descriptor is a valid page descriptor;

- if so, check the type of the old descriptor; if it is a page

descriptor, too, carry its USED/MODIFIED bits over to the new

descriptor. If it is a table descriptor, carry its MODIFIED bit

over. This step is required to keep the U/M bits consistent in case

the user wants to install a new descriptor while pages haven't been

swapped out/read out yet.

- write the new descriptor back to memory.

- push the cache line containing the new descriptor so the MMU will

really find it.

- check whether the logical address that is maintained by this

descriptor is the zero-page; if so, the descriptor address must be

placed in ctxt->ctx_ZeroPage for the exception handler

Note that the context you get passed in need not to be the active

context.

Note further that this call expects a valid page descriptor active

already; in case you want to install a page descriptor into a not

yet valid array of descriptors, use InstallNewPage() instead.

BUGS

SEE ALSO

InstallNewPage(), ForbidDMACache()

mmu.resource/InstallNewPage			mmu.resource/InstallNewPage

NAME

InstallNewPage() - Install a page descriptor into the MMU tree

SYNOPSIS

InstallNewPage( level, tablesize, dest, descr, data, addr, ctxt, mmubase);

d2	   d1	   a2    d0     d4    a0    a5    a6

void InstallNewPage( WORD, ULONG, ULONG *, ULONG, ULONG, ULONG,

struct IMMUContext *, struct MMUBase *);

FUNCTION

Install a page descriptor into a MMU tree; keep care about DMA

disable counters. Install new page data, adjust the context.

INPUTS

level	-	level of the MMU tree where the descriptor shall be

installed. Zero is level A.

tablesize -	size of the descriptor table at this level in entries

dest	-	physical address where to install the descriptor to

descr	-	the descriptor to install

data	-	secondary page data to install

addr	-	logical address that is described by this descriptor

ctxt	-	struct IMMUContext this descriptor is part of

mmubase	-	library base pointer of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode. This will be called with all interrupts disabled.

This call must:

- check whether we need to install at page level; if so, DMA disable

counters are available at byte offset "tablesize * 8" from the

descriptor destination address "dest".

- if the DMA counter is found active, the descriptor is found to be

a valid page descriptor with copyback enabled, copyback must be

disabled. This is required for proper support of ForbidDMACache()

as the user may want to install a descriptor while a DMA operation

is running on the same page.

- secondary page data must be written to byte offset "tablesize * 4"

from "dest". This page data consists either of the "property flags"

if the page descriptor is valid or indirect, or on the "user data"

for invalid or swapped non-repairable pages.

- write the new descriptor back to memory.

- push the cache line containing the new descriptor so the MMU will

really find it.

- check whether the logical address that is maintained by this

descriptor is the zero-page; if so, the descriptor address must be

placed in ctxt->ctx_ZeroPage for the exception handler

Note that the context you get passed in need not to be the active

context.

This call does not make any assumptions about the contents of

"dest"; therefore, it does not try to carry over the USED/MODIFIED

bits from there and their contents will be lost resp. replaced by

the contents of the new descriptor. Do not use this call in case

you want to replace an already active page descriptor, consider

InstallPage() instead.

BUGS

SEE ALSO

InstallPage(), ForbidDMACache()

mmu.resource/ReadUMFlags			    mmu.resource/ReadUMFlags

NAME

ReadUMFlags() - read and reset the USED/MODIFIED flags

SYNOPSIS

flags = ReadUMFlags( descr, level, ctxt, mmubase );

a0     d0     a5      a6

ULONG ReadUMFlags( ULONG *, WORD, struct IMMUContext *,

struct MMUBase *);

FUNCTION

Reads the USED/MODIFIED flags of the indicated descriptor, returns

them and clears them in the descriptor for the next go. This call

shall be used to drive a virtual memory engine.

INPUTS

descr	-	address of the descriptor to be read

level	-	level of the descriptor within the MMU tree. Level A

is indicated by zero.

ctxt	-	pointer to the IMMUContext this descriptor is part of.

mmubase	-	base of the mmu.library

RESULTS

a combination of MAPP_USED and/or MAPP_MODIFIED depending on whether

the memory page handled by the descriptor has been read, or touched

since the last call of this function or InstallNewPage().

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode. This will be called with all interrupts disabled.

This call must:

- check whether the descriptor is a page or a table descriptor or

invalid or indirect. Table descriptors only carry the USED and

not the MODIFIED bit; invalid or indirect descriptors do not carry

any status information.

- read the descriptor and extract the apropriate bits. These are

no bits at all for indirect or invalid descriptors.

- push the cache line containing the descriptor back to memory.

BUGS

SEE ALSO

InstallNewPage()

mmu.resource/SetIndirect			    mmu.resource/SetIndirect

NAME

SetIndirect() - install an indirect page descriptor

SYNOPSIS

SetIndirect( where, logical, descr, mmubase );

a0       a1     d0	a6

void SetIndirect( ULONG *, ULONG, ULONG, struct MMUBase * );

FUNCTION

Installs a new user-generated indirect descriptor at the

supplied address and flushes the caches accordingly.

INPUTS

where	-	physical address into which the descriptor shall go

logical	-	logical address of the page that is handled by the

descriptor or (ULONG)(~0) if it handles more than

one or an unknown logical address

descr	-	the descriptor to install

mmubase -	base of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

This call

- checks the logical address against ~0. If it is, the ATC is

flushed completely, the descriptor is installed, and the

cache line containing the ATC is pushed, too.

- if it is not, only the ATC for the target logical address is

pushed, the descriptor is installed, the cache line is pushed,

and the ATC is flushed again.

BUGS

SEE ALSO

SetIndirectArray()

mmu.resource/SetIndirectArray			mmu.resource/SetIndirectArray

NAME

SetIndirectArray() - install an array of indirect descriptors at once

SYNOPSIS

SetIndirectArray ( where, descr, num , mmubase );

a0	   a1    d0	  a6

void SetIndirectArray ( ULONG *, ULONG *, ULONG, struct MMUBase *);

FUNCTION

Installs an array of user generated descriptors at once and handles

caches accordingly.

INPUTS

where	-	physical destination address where the descriptors

shall go

descr	-	logical address of the source array to take the

descriptors from

num	-	number of descriptors to install, i.e. size of the

array in entries

mmubase	-	base address of the mmu.library

RESULTS

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

This call shall try to find the most efficient way to install the

passed descriptors; for example, it might want to install as many

descriptors at once before pushing their cache lines; this call

must also flush the ATC when done.

BUGS

SEE ALSO

SetIndirect()

mmu.resource/WithoutMMU				    mmu.resource/WithoutMMU

NAME

WithoutMMU() - call a small user interface with the MMU disabled

SYNOPSIS

res = WithoutMMU( userprocptr, mmubase );

d0		      a5	 a6

ULONG WithoutMMU( APTR userproc, struct MMUBase *);

FUNCTION

Calls the supplied user routine with the MMU and all interrupts

disabled; the user routine must return with RTS and may provide

a result code in register d0 that is passed out again.

INPUTS

userprocptr -	physical address of the user routine to be

called

mmubase	    -	base address of the mmu.library

RESULTS

res -	whatever was left in regster d0 by the user routine.

NOTES

Note that a6 points to the mmu.library base, not to the resource

base.

This is always called from supervisor mode; do not attempt to call

it from user mode.

This routine is, in fact, completely register transparent; it

returns all the registers that the user routine left in the CPU.

Note however that the user routine might not be able to do very

much: Due to the disabled MMU, some system structures, or even the

complete Os might be absent.

The implementation is required to save back some of the MMU state,

push all caches, disable the interrupts and the MMU, call the user

function, flush the ATC and caches, and restore the MMU state again.

It must not touch any registers - or rather, must restore them if it

does.

BUGS

Requires at least the supervisor stack to map the same with and

without the MMU. There is no much hope otherwise.

SEE ALSO

mmu/WithoutMMU()

mmu.resource/Exception					mmu.resource/Exception

NAME

Exception()	-	Handle a bus error exception

SYNOPSIS

Exception();

Exception();

FUNCTION

This is the complete access error handler of the MMU/CPU combination

that is handled by this mmu.resource. You must not call this function;

rather, the mmu.library will install the function vector of this

routine directly into the vector base of the CPU. It is the job of this

routine to supply the ExceptionData structure to user exception

handlers, to provide suitable access to the CPU pipelines or provide an

emulation thereof, and to emulate accesses into the zero page,

including AbsExecBase, if the active context allows so.

All this makes this function maybe the most trickiest one of the

mmu.library combo.

INPUTS

None, except the complete MMU/CPU status when this is called on a

bus error. Note that this routine gets no pointer to the mmu.library

passed in, even though it is required. Use CheckMMUInterface()

above to keep a local copy of the library base somewhere as it is

needed.

RESULTS

None, except that the provided user exception handlers must be

handled consistently; if none of them is willing to accept this

access error, the system access error handler must be invoked,

causing the infamous "Guru".

NOTES

This is the beasty one.

This function shall do the following:

- Check for the special branch cache prediction error for the 68060.

If found active, the branch cache must be flushed.

- Fetch the active context from the mmu.library base. Check whether

this is a user or supervisor access, select the proper context.

- Check for long-word reads from AbsExecBase; if found, and the

context allows these reads, provide a cached copy of AbsExecBase.

Do *NOT* attempt to read it from the zero page or you end up in an

infinite loop.

Several strategies exist to provide AbsExecBase access emulation:

- place it in the CPU input pipeline if the possibility

exists (68020, 68030 only),

- emulate the offending instruction by hand. This is easy

for simple "move.l" cases.

- make the zero page available, enable tracing, let the

access happen, catch the trace exception, disable the

zero page again. The active context contains a pointer

to the zero page descriptor in ctxt->ctx_ZeroPage you

may find useful to modify.

- Check for physical bus error cases or read/modify/write cycles

you need not to handle. If found, restore all registers, and

jump into MMUBase->mulib_OldException(). This will call the system

guru.

- If you decide to handle the access error:

Allocate the ExceptionData structure in

MMUBase->mulib_ExceptionData by setting mmuf_busy in

MMUBase->mulib_Flags1. If you find this bit "on" already,

a double bus fault happened and you better go guru.

- Fill in the ExceptionData structure; keep care about special

cases as MOVE16, MOVEM with predecrement, FMOVE.X, FMOVE.D when

computing the affected logical addresses.

- Check whether the fault goes into the readable part of the

zero page by comparing against ctxt->ctx_LowMemoryLimit. If the

address is larger than this limit, access must be emulated. One

approach to handle this is to enable tracing, enable the zero page

access by modifying the zero page descriptor pointed to by

ctxt->ctx_ZeroPage, re-run the access, catch the trace exception,

disable zero page access again.

- Check for read exceptions. If found, call the following private

function of the mmu.library with the ExceptionData pointer in a0:

fatal = CallExceptionHandlers( excdata );

d0				 a0

BOOL CallExceptionHandlers( struct ExceptionData *);

It returns TRUE for a fatal exception; restore registers and

jump into MMULib->mulib_OldException() to invoke the guru.

Otherwise, check ExceptionData->exc_Flags how to continue.

- If (exd_Flags & exdf_call), a user supplied function must be

called in user mode, possibly interrupting your access error

handler. Luckely, no need to mess with this yourself as the

mmu.library can do this for you with the following private

call:

SubUser( excd );

a5

void SubUser( struct ExceptionData *exc );

- If (exd_Flags & exdf_readback), the user provided data for the

CPU pipeline that shall be read. Check for instruction access

errors; if found, go guru and don't try to handle them. Check

for REPAIRABLE as well. If not available, go guru as well.

If you do not have access to the CPU pipeline, you need to emulate

the access to fill the user data: ctxt->ctx_ReadBackPage provides

the physical address of a page that is set aside for this mess.

Fill this page with the user supplied data in the proper alignment,

and modify the descriptor at (not pointed to!)

ctxt->ctx_InvalidIndirect with the proper descriptor. All

"repairable" page descriptors are indirect and point to this

descriptor. Enable tracing, let the access happen - now to the

ctx_ReadBackPage, and catch the trace exception. Disable tracing,

and access to the ReadBackPage, and continue.

- Check for write exceptions. If found, check the properties of

the descriptors that are accessed. If they indicate "MAPP_ROM",

the access must be tolerated without modifying the original page.

If you cannot abort the write by fiddling the CPU pipeline, you

need to provide an emulation again: Replace the involved pages

(at most two due to misalignment) by descriptors pointing to

the two pages at ctx->ctx_RomPages, and keep the original

descriptors in a temporary. Copy original page data over to the

ROM pages of the context, enable tracing, let the access happen.

In the trace exception, restore the original descriptors and

disable tracing.

- Check whether the user likes to see the data that was written

out by checking for MAPP_REPAIRABLE; if so, either try to

supply from the CPU pipeline if available, otherwise you are

in a mess:

- Replace again the pages by the context ROM pages, enable

tracing, store the complete CPU state, let the access happen,

catch the trace exception. Call the user handler/user routines

in here and provide the written out data from the ROM pages

that got modified at the address you recorded before. In case

the user wants to repair the page itself by not setting the

exdf_writecomplete flag in ExceptionData, you are now really in

a mess:

Restore the complete CPU state to what you found during the

access error, restore the descriptors and re-start from the point

where the access error happens.

- If the user does not want the read-out data, call user handlers

and SubUser() within the access error handler directly.

- If the user signaled re-run by not setting exdf_writecomplete,

let the access happen by restoring all registers and running

into an RTE. If not, you still need to provide a destination

where the write goes into; replace the affected descriptors

with the ROM pages of the context, install trace vector,

let the access happen and clean up in the trace exception.

Good luck, I'm with you in spirit.

BUGS

Much more needs to be said about this; the implementation depends

heavely on the CPU/MMU programming model, and there are little

general guidelines I can give here. Test carefully!

SEE ALSO

Exception.doc, mmu/context.h