AudioSelect -- Control the Picasso IV audio output

==================================================

In its basic configuration, the Picasso IV carries the signal of one out of

four audio input sources on its audio connector. With "AudioSelect" you can

control which of the four sources will be made available.

1. Running the program

AudioSelect runs both from Shell and from Workbench. It is recommended to

put the program into the "WBStartup" drawer of your system partition to have

it handy.

1.1 Parameters and icon tool types

The features of the program can be controlled both via Workbench and Shell.

In both cases, the parameters and tool types bear the same names and serve

the same purposes:

CX_Priority=<-128..127>

CX_PopKey=<Specifier>

CX_PopUp=<Yes or No>

These are the common parameters every Commodities tool offers.

Unless you provide a different configuration by editing the

tool types or changing Shell parameters the program will default

to use the following configuration:

CX_Priority=0

CX_PopKey=alt f10

CX_PopUp=Yes

This means that after the program has been started, it will

immediately open its settings window (CX_PopUp=Yes) and will

show its settings window every time you hold down any [Alt]

key and press the [F10] key (CX_PopKey=alt f10).

Board=<1..10>

Up to ten Picasso IV expansion boards are supported in one single

Amiga computer. With this parameter you choose which board the

AudioSelect program should act upon. If the parameter is omitted,

the program will default to talk to board #1.

Mute=<Yes oder No>

You can switch off audio output completely with this parameter.

To switch is off use MUTE=YES, to switch it back on use MUTE=NO.

If this parameter is omitted the state of the audio output will

be left unchanged.

Source=<Amiga, AVModule, LineIn oder CD>

This parameter selects the audio signal source to use

Amiga    - Amiga audio signal

AVModule - Audio signal of the AV module

LineIn   - Selects the external audio input connector

CD       - Selects the internal audio input connector,

this is where you would connect your

CD-ROM drive to

If this parameter is omitted, the current audio source will

be left unchanged.

Set

Normally, the program will stay in memory after you have

started it and the audio configuration has been changed.

However, if you use the "SET" parameter, the program will

exit immediately after changing the audio configuration.

1.2 The settings window

You can also mute the audio signal and change the signal source via the

settings window. The "Mute" button toggles whether the signal will be muted

or not and the "Source" switch selects the signal source.

To close the settings window whilst keeping the program in memory, click the

window close gadget or use the "Hide" menu item. To terminate the program,

use the "Quit" menu item or use the "Exchange" tool to "Remove" the

AudioSelect program.

2. Examples

2.1. Usage from Shell

To feed the input from the external audio connector through to the

output, enter the following:

audioselect set source linein

To mute the audio output:

audioselect set mute

2.2. Usage from Workbench

This requires that you edit the icon tooltypes of the AudioSelect

program.

To feed the input from the external audio connector through to the

output, edit the "SOURCE" tooltype and add the "SET" tooltype so

they look like this:

SOURCE=LineIn

SET

Then double-click on the icon.

To mute the audio output, edit the tooltypes like this:

MUTE=Yes

SET

Then double-click on the icon.